ShopBot® 2011 STEM Neck Tooling

For ease of use, the tool paths for both the right-hand and left-hand necks are progressive, identified by “Step #” where # is 1, 2, 3, and 4.

* All tooling requires at least 1” of usable machining bit exposed from the chuck/collet *

- Tool paths containing “Step 1” and “Step 2” require a ¼” Straight-end mill cutter.

- Tool paths containing “Step 3” require a 7/64” diameter drill. (this step is not crucial and can be performed by hand during the assembly process)

- Tool paths containing “Step 4” require a 1/2” Ball-end mill cutter.

Tool paths containing “Step 1” are done on the front side of the neck blank; all other tool paths occur on the back side.

Machining notes: The standard 32” x 4-1/2” x .750” neck blank is used for this tooling collection. All touch-offs are done at the top of the neck blank, and maximum depth of cut is 0.800” to ensure complete machining. The recommended mounting post spread is a minimum of 30.0”.

Left-hand Neck Tooling

In order to accommodate the left-hand neck tooling, it is required that the X-unit value be set to negative in the ShopBot® software. (Accessed by selecting “Values” then “[U]nit” from Shopbot’s control software main screen menu)

- Note that your specific value is unique to your particular model of ShopBot. Do not modify the value of this number, only negate it!

- Be sure to set the sign of the value back once you are done with the left-hand neck machining.

- Also be aware that X-axis table movements will be reversed during this time. I.E.: A command of JX 7 will move the table 7” in the direction opposite to normal!

Tried-and-True-advice: Write down your X, Y, and Z unit values before you modify them. That way you can easily reset them should they get corrupted.

Neck Design and Tooling:

Doug Hunt, Southern Wells High School

Poneto, Indiana

Documentation:

Philip Smith

Modeler/CNC Programming Tech

philip.smith8175@sinclair.edu

Sinclair Community College

Dayton, Ohio