ShopBot® 2011 STEM Neck Tooling

For simplicity, the tool paths for both the right-hand and left-hand necks are progressive, identified by “Step #” where # is 1, 2, 3, and 4.

* All tooling requires at least 1” of usable machining bit exposed from the chuck/collet *

- Tool paths containing “Step 1” and “Step 2” require a ¼” end-mill cutter.

- Tool paths containing “Step 3” require a 7/64” diameter drill. (this step is not crucial and can be performed by hand during the assembly process)

- Tool paths containing “Step 4” require a 1/2” end-mill cutter.

Tool paths containing “Step 1” are done on the front side of the neck blank, while all other tool paths occur on the back side.

Machining notes: The standard 32” x 4-1/2” x .750” neck blank is used for this tooling collection. All touch-offs are done at the top of the neck blank, and maximum depth of cut is 0.800” to ensure complete machining. The recommended mounting post spread is a minimum of 30” center-to-center.

Left-hand Neck Tooling

In order to accommodate the left-hand neck tooling, it is required that the X-unit value be set to negative in the ShopBot® software. (Accessed by selecting “Values” then “[U]nit” from Shopbot’s control software main screen menu)

- Note that your specific value is unique to your particular model of ShopBot. Do not modify the value of this number, only negate it!

- Be sure to set the sign of the value back once you are done with the left-hand neck machining.

- Also be aware that X-axis table movements will be reversed during this time. A JX 7 will move the table 7” in the opposite direction than normal!

Tried-and-True-advice: Write down your X, Y, and Z unit values before you modify them. That way you can easily reset them should they get corrupted.

Neck Tooling and Design by Doug Hunt, Southern Wells High School

Compiled by Philip Smith

Modeler/CNC Programming Tech

philip.smith8175@sinclair.edu

Sinclair Community College

Dayton, Ohio